COMPARISON OF FLSM AND VLSM

Cisco Packet Tracer Networking Assignment

# 1. Aim

To design and compare Fixed Length Subnet Masking (FLSM) and Variable Length Subnet Masking (VLSM) for five departments requiring 70, 35, 18, 9 and 5 hosts, and to compare address utilisation and observed ping performance.

# 2. Network Requirements

| Department | Required hosts |
| --- | --- |
| Department 1 | 70 |
| Department 2 | 35 |
| Department 3 | 18 |
| Department 4 | 9 |
| Department 5 | 5 |

# 3. FLSM Address Allocation

In FLSM, every department receives the same subnet size. The largest department requires 70 hosts, so a /25 subnet (255.255.255.128) is used for every department. Each /25 contains 128 total addresses and 126 usable host addresses.

| Department | Network / Prefix | Usable host range | Broadcast | Usable hosts |
| --- | --- | --- | --- | --- |
| 1 | 192.168.0.0/25 | 192.168.0.1–126 | 192.168.0.127 | 126 |
| 2 | 192.168.0.128/25 | 192.168.0.129–254 | 192.168.0.255 | 126 |
| 3 | 192.168.1.0/25 | 192.168.1.1–126 | 192.168.1.127 | 126 |
| 4 | 192.168.1.128/25 | 192.168.1.129–254 | 192.168.1.255 | 126 |
| 5 | 192.168.2.0/25 | 192.168.2.1–126 | 192.168.2.127 | 126 |

Theoretical parent block for this table: 192.168.0.0/22. The Packet Tracer FLSM ping screenshot shows PC1 testing 192.168.2.2.

# 4. VLSM Address Allocation

In VLSM, each department receives the smallest subnet that supports its host requirement. The following is a compact example allocation within 192.168.0.0/24.

| Department | Network / Prefix | Usable host range | Broadcast | Usable hosts |
| --- | --- | --- | --- | --- |
| 1 (70 hosts) | 192.168.0.0/25 | 192.168.0.1–126 | 192.168.0.127 | 126 |
| 2 (35 hosts) | 192.168.0.128/26 | 192.168.0.129–190 | 192.168.0.191 | 62 |
| 3 (18 hosts) | 192.168.0.192/27 | 192.168.0.193–222 | 192.168.0.223 | 30 |
| 4 (9 hosts) | 192.168.0.224/28 | 192.168.0.225–238 | 192.168.0.239 | 14 |
| 5 (5 hosts) | 192.168.0.240/29 | 192.168.0.241–246 | 192.168.0.247 | 6 |

Important: the VLSM Packet Tracer screenshot provided shows a ping to 10.0.1.242. Therefore, use the network addresses from your saved Packet Tracer VLSM file if they differ from the example allocation table above. The prefix sizes and address utilisation calculations remain the same.

# 5. Address Utilisation Comparison

| Metric | FLSM | VLSM | Comparison |
| --- | --- | --- | --- |
| Subnet prefixes | /25 for all | /25, /26, /27, /28, /29 | VLSM varies subnet size |
| Total allocated addresses | 640 | 248 | 392 fewer with VLSM |
| Usable host addresses | 630 | 238 | VLSM has less unused capacity |
| Required hosts | 137 | 137 | Same requirements |
| Unused usable capacity | 493 | 101 | 392 fewer unused usable addresses |
| Allocated address-space reduction | — | 61.25% | 392 ÷ 640 × 100 |

Observation: VLSM allocates 248 addresses compared with 640 for FLSM, saving 392 addresses (61.25%).

# 6. Network Performance Measurements

Three ping commands were run from PC1 to a destination PC in each configuration. The values below are transcribed from the supplied Packet Tracer screenshots.

| Method | Run | Sent | Received | Packet loss |
| --- | --- | --- | --- | --- |
| FLSM | 1 | 4 | 3 | 25% |
| FLSM | 2 | 4 | 4 | 0% |
| FLSM | 3 | 4 | 4 | 0% |
| VLSM | 1 | 4 | 3 | 25% |
| VLSM | 2 | 4 | 4 | 0% |
| VLSM | 3 | 4 | 4 | 0% |

Combined results for each method: 12 packets sent, 11 received and 1 lost. Overall packet loss = (1 ÷ 12) × 100 = 8.33%. Packet Tracer displayed minimum, maximum and average round-trip times of 0 ms in both configurations, while individual replies were shown as time<1ms.

Observation: both methods produced the same results in these tests. The first ping in each set timed out, while the next two ping commands succeeded completely. These measurements do not demonstrate a difference in packet loss or displayed RTT.

# 7. Conclusion

The experiment shows that VLSM is more efficient in address allocation for the given five departments. FLSM allocates 640 addresses, while VLSM allocates 248, saving 392 addresses or 61.25%. In the ping tests, both configurations had 11 successful replies out of 12 packets and a displayed average RTT of 0 ms. Thus, the collected tests show improved address utilisation with VLSM, but do not show a measurable network-performance difference.

- s